Voice mailbox

Retrieval

Guide

29 Oct 2008                                                                                                                    Version 1.01

Sunny Line Limited © 2007-2008

Trademarks

Digium and Asterisk are registered trademarks of Digium, Inc.

http://www.digium.com

All other trademarks are property of their respective owners.

Table of Content

1. Introduction	1

1.1 Feature Codes To Access Voicemail Menu	1

2. Voice Mailbox Retrieval Menu	2

3. Voice Mailbox Sizing	3

# 1. Introduction

This short document describes how to retrieve voice message from mailbox and operations in the mailbox. If voice mail notification with email attachment is configured and voice messages are deleted after email notification, there is no need to access voice mailbox described in this document. Please consult system support or administrator for site-specific configuration.

By default, there are two feature codes for voice mailbox retrieval; “*98” and “*97”. The latter is used when the dialing extension number is the same as mailbox number. In some site implementation, different feature codes or special extensions may be configured for voice mailbox retrieval; please consult system support or administrator for confirmation.

Target audiences are call center supervisors, agents or backend staff who needs to retrieve messages from voice mailboxes.

## 1.1 Feature Codes To Access Voicemail Menu

| No. | Feature code | Remark |
| --- | --- | --- |
| 1 | *97 | The dialing extension will be used as voice mailbox number; thus, no prompting for voice mailbox number input |
| 2 | *98 | The dialing extension will be ignored; thus, instruction prompting for voice mailbox number is present |
| 3 | *98nnnn | Numbers (nnnn in this example) followed by the access code will be used as voice mailbox number; thus, no prompting for voice mailbox number input |

# 2. Voice Mailbox Retrieval Menu

| No. | Step | Response / Remark |
| --- | --- | --- |
| 1 | Dialing stage |  |
| 1.1 | On any Asterisk phone, dial one of them *98 *98nnnn *97 | Where nnnn = voice mailbox no. dialing extension as voice mailbox no. |
| 2 | Login stage |  |
| 2.1 | Message: vm-login | For feature code “*98” only; input mailbox number via DTMF key |
| 2.2 | Message: vm-password | Input password of mailbox number |
| 2.3 | If failed, messages: vm-incorrect-mailbox vm-password | Input password of mailbox number |
| 2.4 | If succeeded, messages: vm-youhave [n] vm-INBOX vm-messages |  |
| 3 | Voice mailbox menu |  |
| 3.1 | 1 – Read voicemail messages First voice message 3 Advanced option (not enabled) 4 Play previous message 5 Play current message 6 Play next message 7 Delete current message 8 Forward message to another mailbox 9 Save message in a folder * Help; during msg playback: Rewind # Exit; during msg playback: Skip forward | - required “enhanced voicemail” - Disabled if no previous message - Disabled if no next message |
| 3.2 | 2 – Change folder 0 Switch to new message 1 Switch to old message 2 Switch to work message 3 Switch to family message 4 Switch to friends message |  |
| 3.3 | 3 – Advanced option (not enabled) | - required “enhanced voicemail” |
| 3.4 | 0 – Mailbox option 1 Record your unavailable message 2 Record your busy message 3 Record your name 4 Record your temporary message 5 Change your password * Return to the main menu After recording a message (incoming message, busy/unavail greeting, or name) 。1 - Accept this recording 。2 - Listen to it 。3 - Re-record your message 。0 - Reach operator(1) (not available when recording greetings/name) |  |
| 3.5 | * - Help |  |
| 3.6 | # - Exit |  |

# 3. Voice Mailbox Sizing

The maximum number of voice mails and maximum duration of a voice mail are configured by global value and by per mailbox basis. Users should consult system support/administrator or software supplier for site specific configuration. Disk space sizing for voice mail messages is around 2KB per second.

Sizing calculation example

| No. | Item | QTY | Remark |
| --- | --- | --- | --- |
| 1 | Max no. of voice mails | 100 | Inbox or new message If maximum number is reached, “mailbox full” message will be announced for any voice mail deposit attempt. |
| 2 | Max duration (sec) / message | 180 |  |
| 3 | Max no. of Old messages | 100 | Same maximum number as Inbox; moved after being listened but not deleted explicitly. If Old message folder is full, user will NOT be notified. Any listened Inbox/new message will be deleted rather than moved to Old message folder. Old folder messages must be deleted manually |
| 4 | Max duration (sec) | 36,000 | (100 + 100) msg X 180 sec/msg |
| 5 | Max disk space required | 72MB | 36,000 sec X 2KB / sec |

#Two folders, Inbox/new and Old, are considered in above calculation.