Asterisk & Customized

Telephony Objects

Configuration Guide

29 Oct 2008                                                                                                                    Version 1.04

Sunny Line Limited © 2007-2008

Trademarks

Digium and Asterisk are registered trademarks of Digium, Inc.

http://www.digium.com

FreePBX is a registered trademark of Atengo LLC.

http://www.freepbx.org

Microsoft, Windows, Window 2000 / 2000 Professional / XP Home / XP Professional / 2003 server MS SQL 2005 Server, MS Visual Studio 2005 and their components are registered trademarks of Microsoft Corporation.

http://www.microsoft.com

MySQL is a registered trademark of Sun Microsystems, Inc

http://www.mysql.com

trixbox is a registered trademark of Fonality, Inc.

http://www.trixbox.com

All other trademarks are property of their respective owners.

Table of Content

1. Introduction	1

1.1 Configuration objects summary	1

2. IAX Extension	2

2.1 Add new IAX extension	4

2.2 Modify / Delete IAX extension	8

3. Agent Profiles	9

3.1 Add agent profile entry	11

3.2 Modify agent profile	12

3.3 Delete agent profile	14

3.4 Upload agent profile to Asterisk	16

4. ACD Queue	17

4.1 Add queue	19

4.2 Modify / delete queue	23

5. Queue Attributes for Historical Reports	24

5.1 Add queue entry	26

5.2 Modify queue entry	27

5.3 Delete queue entry	28

6. Grouped Extensions (Outbound historical reports)	29

6.1 Add group name / extension	31

6.2 Delete group name/extension	32

7. Inbound Routes	34

7.1 Add incoming route	36

7.2 Modify / delete incoming route	40

8. Outbound Routes	41

8.1 Trunks object	41

8.1.1 Add Trunk	42

8.1.2 Modify / delete Trunk	44

8.2 Outbound route module	45

8.3 Add outbound route	46

8.4 Modify / delete outbound route	49

9. SIP Extension (optional)	50

9.1 Add SIP extension	50

9.2 Modify / Delete SIP extension	54

10. Features codes configuration	55

10.1 Silence monitor customization	57

Appendix A – Configuration Tables Maintenance Data Flow Diagram	58

# 1. Introduction

This document goes thought a number of steps in Asterisk to configure necessary telephony objects in a typical call center environment. Objects include extension (IAX type), Agent login-ID, ACD queue, customized tables for historical reports, inbound route and outbound route. Features such as announcements, IVRS and miscellaneous call flow control related objects will not be covered.

It is assumed that installation and configuration of Asterisk and FreePBX are successfully completed. External voice interface (T1/E1 card or IP gateway) has been properly configured for inbound and outbound calls.

Target audiences are system administrators and authorized call center supervisors.

## 1.1 Configuration objects summary

| No | Object name | Module location |
| --- | --- | --- |
| 1 | IAX extension (for iaxSphone) (optional DID extension) | http://<asterisk-ip>/admin FreePBX -> [Setup] -> [Extensions] |
| 2 | Agent profiles **IAX extensions and agent IDs should be identical for free-seating! | http://<asterisk-ip>/cfgmaint [3. Agent profiles configuration] |
| 3 | ACD Queue (mapping to hotline DNIS) | http://<asterisk-ip>/admim FreePBX -> [Setup] -> [Queues] |
| 4 | Queue service level/attributes | http://<asterisk-ip>/cfgmaint [1. Queue service level/attributes configuration] |
| 5 | Grouped extensions (for outbound calls) ** replicate queue/members in queue mapping | http://<asterisk-ip>/cfgmaint [2. Grouped extensions configuration] |
| 6 | Inbound routes | http://<asterisk-ip>/admim FreePBX -> [Setup] -> [Inbound Routes] |
| 7 | Outbound routes | http://<asterisk-ip>/admim FreePBX -> [Setup] -> [Outbound Routes] |
| 8 | Optional SIP extension (for SIP hard-phone or soft-phone) | http://<asterisk-ip>/admin FreePBX -> [Setup] -> [Extensions] |
| 9 | Feature codes settings | http://<asterisk-ip>/admin FreePBX -> [Setup] -> [Feature Codes] NB: read notes on silence monitoring and voice logging features carefully |

# 2. IAX Extension

IAX (Inter-Asterisk-eXchange) protocol 2 extension type should be configured for iaxSphone which integrates agent login/logout/mode. IAX extension is maintained in FreePBX setup module.

IAX can be configured manually in /etc/asterisk/iax_custom.conf; it is out of scope in this manual.

Startup URL in web browser: http://<asterisk-ip>/admin

- <asterisk-ip> = IP address of Asterisk server or host name if it can be resolved in intranet environment

- Username / password are required to access the web page

Top horizontal tabs

Category / module

LHS tabs

The FreePBX main administrator page is illustrated as above.

- In “Top horizontal tabs”, the [Admin] tab is to configure Asterisk objects

  - Other tabs are not used; they are replaced by customized modules

- LHS (Left-Hand-Side) tabs appear in [Admin] tab (of Top horizontal tabs) only

  - For normal daily operations, only the [Setup] tab will be referred

- A list of Category / Modules is listed under the [Setup] tab

## 2.1 Add new IAX extension

To add a new IAX extension

- Click [Extensions] module under {Basic} category

- Click [Add Extension] in the list of RHS if current mode is modification

- In the Device pull-down menu, select [Generic IAX2 Device]

- Click [Submit] button

Parameters group name

Parameter names/values

Online help description for many fields are available when pointing cursor over the parameter field name. This document will briefly covers common fields to support normal operation. If description is left blank, the field is optional or default value

Group: Add Extension

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | User Extension | Mandatory; the extension number to dial to reach this user |
| 2 | Display Name | Caller ID name (not number) for calls from this user |
| 3 | CID Num Alias | NB: leave this field blank or recorded voice file name will be incorrect for searching! |
| 4 | SIP Alias |  |

Group: Extension Options

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Direct DID | Set the Direct Inwards Direct number if this extension is configured to be reached by external party directly For normal agents, this field is left empty |
| 2 | DID Alert Info |  |
| 3 | Music on Hold | [default] in pull-down list |
| 4 | Outbound CID |  |
| 5 | Ring Time | [Default] in pull-down list |
| 6 | Call Waiting | [Disabled] in pull-down list for ACD agent’s extension |
| 7 | Emergency CID |  |

Group: Device Options

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | secret | “abc123” (the ‘default’ value in this document) |
|  | Fields/parameters in modification mode; non-default values are bolded |  |
| 2 | notransfer | no |
| 3 | context | from-internal |
| 4 | host | dynamic |
| 5 | type | friend |
| 6 | port | 4569 |
| 7 | qualify | no |
| 8 | disallow |  |
| 9 | allow |  |
| 10 | dial | IAX2/nnnn where nnnn = the current extension number |
| 11 | accountcode |  |
| 12 | mailbox | nnnn@device where nnnn = the current extension number |

Group: Fax Handling

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Fax Extension | [disabled] in pull-down list |
| 2 | Fax Email |  |
| 3 | Fax Detection Type | [Non] in pull-down list |
| 4 | Pause after answer | 0 |

Group: Privacy

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Privacy Manager | [No] in pull-down list |

Group: Recording Options

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Recording Incoming | [Always] in pull-down list |
| 2 | Recording Outgoing | [Always] in pull-down list |

Group: Voicemail & Directory

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Status | [Enabled] in pull-down list to create mailbox for the extension; otherwise, [Disabled] |
|  |  | If [Enabled], set below parameters |
| 2 | Voicemail Password | Password to access the voice mail system; must be all numbers |
| 3 | Email Address | Optional email address to get voice mail notification |
| 4 | Pager Email Address | Optional pager email address to get voice mail notification |
| 5 | Email Attachment | “yes/no”; if email address is given, recorded voice mail file can be attached in the voice mail notification |
| 6 | Play CID | “yes/no”; play Call ID in voicemail retrieval operations |
| 7 | Play Envelope | “yes/no”; play deposited voicemail date/time in retrieval operations |
| 8 | Delete Vmail | “yes/no”; if email address is given, it controls if the voice mail should be deleted after sending notification mail. NB: the option does not check if “Email Attachment” is set! |
| 9 | VM options | Additional voice mail options separate with pipe (¦) Options specified here may override global values; samples parameters are listed below. Maximum options allowed is 9 (nine). maxmsg=100 ; max number of messages maxmessage=180 ; max seconds of vmail message review=no ; Allow sender to review/rerecord their message before saving it operator=no ; Allow caller to press 0 nextaftercmd=yes ; Automatically play next message when current message is deleted or saved |
| 10 | VM Context | Leave it as “default”, the default value |
| 11 | VM X LocatorTM | [Disabled] in pull-down menu |

Click the [Submit] button at the bottom of page

To make the change effective immediately,

Click [Apply Configuration Changes] at the top of page; the “Apply” button is usually clicked after finishing a batch of changes.

In the window, “Reloading will apply all configuration changes made in FreePBX to your PBX engine and make them active”, click [Continue with reload] to proceed.

## 2.2 Modify / Delete IAX extension

(2a) Current working extension

Modify parameters and values as describe in last section (i.e., add IAX extension)

(1) Click one of existing extension on the RHS list

(2b) Current working extension

Click this area to Delete the current extension

- After modification or deletion, click the [Submit] button at the bottom of page

- To make the change effective immediately, click [Apply Configuration Changes] at the top of page

# 3. Agent Profiles

Agent profiles (agent login ID, agent password and agent name) can be edited directly in Asterisk configuration. A customized web-based configuration is made to ease normal call center operations.

Startup URL in web browser: http://<asterisk-ip>/cfgmaint

- <asterisk-ip> = IP address of Asterisk server or host name if it can be resolved in intranet environment

- Cookie must be enabled since session is required

- Username / password are required to access the web page

- Click [HELP] for online help description if needed

- Enter valid [Username] and [Password] values

- Select (x) 3. Agent profiles configuration as shown

- Click [Submit] button to proceed

- For fields and control button description, please return to main menu page and click [HELP]

- After finishing add/modify/delete a batch of agents, the [Update to Asterisk] must be performed to upload agent profiles from database table to Asterisk configuration file.

## 3.1 Add agent profile entry

Fields/button area to add new agent profile

- After entering values, click [Add] button to execute

Result of add-operation will be displayed in this area

If add-operation is successful, total number of records is incremented by 1

## 3.2 Modify agent profile

- Modify fields of an agent; “Agent ID” field is read-only

- Select the “radio” button for the modified agent; only one and only agent can be modified at one instance

- Click the [Update] button to proceed

Result of modify/delete-operation will be displayed in this area

## 3.3 Delete agent profile

- Check one or multiple entries to be deleted in [Delete] column

- Click the [Update] button to proceed

Result of modify/delete-operation will be displayed in this area

- Total number of records is decremented by 3

## 3.4 Upload agent profile to Asterisk

After finish a batch of add/modify/delete operations, records in database table must be uploaded to Asterisk to make changes effective. After clicking [Upload to Asterisk] button, following window will be prompted.

Click [OK] to continue or [Cancel] to abort

A notification message page is displayed; in this example, 18 records are uploaded.

Messages in “Notes” are reminder for system administrators!

Click [Return] to go back to previous menu

# 4. ACD Queue

Queue objects can be configured in FreePBX  (Admin -> [Setup] -> [Inbound Call Control] -> [Queues]) or in /etc/asterisk/queues_custom.conf and /etc/asterisk/queues_custom_general.conf manually. The latter will not be discussed in this document.

Asterisk queue includes a large set of configurable features to cater different call center specific requirements. This document, however, is not intended to cover all scenarios in term of voice flow control related to Asterisk queue. Reminders will be noted here for reference.

- IVRS, optionally, can be configured “in front of” queue for

  - customer language selection

  - customer identification (requiring backend database lookup integration)

- Default busy announcement language is English

  - A set of default voice files is included in Asterisk

  - separate set of language voice files must be recorded for multi-languages queues

  - Language assignment in call flow is call center dependent

- As in Asterisk 1.2, “rrmemory” ring strategy works as round-robin and are more efficient.

- Option “agent timeout” is the timeout parameter for ring-no-answer when agent does not answer ACD incoming, when auto-answer or forced-in feature is disabled.

- Action for “agent timeout” should be “logout agent” (i.e.., agent will be logged out by Asterisk) to avoid distributing ACD call to the same agent

- Option “retry” parameter in [Queue Options] is another important option to fine-tune.

- Asterisk 1.2 queues apply “polling” method for ready agents. Queues will be suspended for “retry” seconds periodically. For every “retry” seconds, Asterisk starts to scan ready agents for incoming calls waiting in queue

- The value of “retry” should be carefully considered and fin-tuned to optimize queue performance (i.e., TSF) and computer server loading

- Option “wrap-up-time” should be set to non-zero value to avoid next ACD call arriving to the same agent too fast

- [Caller Position Announcements] can be enabled without additional object configuration

- Localized languages require language assignment and corresponding set of voice files

- NB: When announcement is being played, it blocks call distribution to ready agent!

- Before enabling [Periodic Announcements] feature, “System Recordings” file and simple single-node IVR must be configured

- Localized languages require language assignment and corresponding set of voice files

- NB: When announcement is being played, it blocks call distribution to ready agent!

- Refer to section 7 Inbound Routes for diverting inbound call to queue

Notes:

- After clicking [Apply Configuration Changes] button to update queue entries, queue statistics fields will be reset; thus, affecting “ccstat” statistics.

- If “ccstat” statistics are required for normal operation hours, changes should be applied during non-operation time window.

Startup URL in web browser: http://<asterisk-ip>/admin

- Refer to section 2 for login procedure

## 4.1 Add queue

Click this item to [Add Queue]

- Click [Add Queue] on the top of RHS

- Operation mode is changed to “Add Queue”

Online help description for many fields are available when pointing cursor over the parameter field name. This document will briefly covers common fields to support normal operation. If description is left blank, the field is optional or default value

Group: Add Queue

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Queue number | Mandatory; the extension number to dial into queue Note: queue number ‘may’ be non-numeric but some additional manual configurations are required to call redirect when making internal call or transfer |
| 2 | Queue name | Queue name to identify the queue number |
| 3 | Queue password | Optional; password of the queue when agent logs into queue |
| 4 | CID name prefix | Prefix of Caller ID; useful to show hotline / queue number to inbound agent when calls arrive |
| 5 | Static agents | Queue members list of agent IDs, one per line Format: A{agentID}[,{penalty}] A = constant prefix for Agent {agentID} = agent ID value defined in section 3 {penalty} = penalty value of agent in queue; 0 = default Higher value is equivalent to less “skillful” |

Group: Queue Options

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Agent Announcement | [None] |
| 2 | Music on Hold Class | [default] |
| 3 | Ringing tone instead of MOH | [ ] Unchecked or call center specific requirement |
| 4 | max wait time | [Unlimited] or call center specific requirement |
| 5 | max callers | [0] (unlimited) or call center specific requirement |
| 6 | join empty | [Yes] Note: this option has no effect in ‘static agent’ configuration |
| 7 | leave when empty | [No] Note: this option has no effect in ‘static agent’ configuration |
| 8 | ring strategy | [rrmemory] or call center specific requirement |
| 9 | agent timeout | [15 seconds] or call center specific requirement |
| 10 | retry | [3 seconds] or call center specific requirement |
| 11 | wrap-up time | [2 seconds] or call center specific requirement |
| 12 | call recording | [No]  enabled at extension object for inbound/outbound |
| 13 | event when called | [Yes]  must be enabled if real-time Statistics Server for Asterisk (SSFA) program and client applications are running. |
| 14 | member status | [No] |
| 15 | service level | 15  for online queue performance statistics |

Group: Caller Position Announcements

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Frequency | [0 seconds] |
| 2 | Announce Position | [No] |
| 3 | Announce Hold Time | [No] |

Group: Periodic Announcements

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | IVR Break Out Menu | [None] |
| 2 | Repeat Frequency | [0 seconds] |
| 3 | Join Announcement | [None] |

Group: Fail Over Destination

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Queues | (enabled for call center specific requirement only) |
| 2 | Terminate Call | (x) Terminate Call: [Hangup] |
| 3 | Extensions | (enabled for call center specific requirement only) |
| 4 | IVR | (enabled for call center specific requirement only) |
| 5 | Custom App | (enabled for call center specific requirement only) |
| … |  | Additional items will be prompted if defined |
| … |  | Additional items will be prompted if defined |

- Click the [Submit] button at the bottom of page

- To make the change effective immediately, click [Apply Configuration Changes] at the top of page

## 4.2 Modify / delete queue

(2a) Current working queue

Modify parameters and values as describe in last section (i.e., add queue object)

(1) Click one of existing queue object on the RHS list

(2b) Current working queue

Click this area to Delete Queue to remove

- After modification or deletion, click the [Submit] button at the bottom of page

- To make the change effective immediately, click [Apply Configuration Changes] at the top of page

# 5. Queue Attributes for Historical Reports

Queue attributes defined in this section are to provide queue parameters for historical reports generation. Online queue statistics “ccstat” parameter is, however, configured in FreePBX queue extension object.

Queue attributes include [Service Level] answer threshold value and [Short Abandon] threshold value for Telephony Service Factor (TSF) computation. Unit of both parameters are in second.

Startup URL in web browser: http://<asterisk-ip>/cfgmaint

- <asterisk-ip> = IP address of Asterisk server or host name if it can be resolved in intranet environment

- Cookie must be enabled since session is required

- Username / password are required to access the web page

- Click [HELP] for online help description if needed

- Enter valid [Username] and [Password] values

- Select (x) 1. Queue service level/attributes configuration as shown

- Click [Submit] button to proceed

- For fields and control button description, please return to main menu page and click [HELP]

- Queue DN (Directory Number) is the “Queue number” value in FreePBX Queues object.

## 5.1 Add queue entry

Fields/button area to add new queue entry

- After entering values, click [Add] button to execute

Result of add-operation will be displayed in this area

If add-operation is successful, total number of records is incremented by 1

## 5.2 Modify queue entry

- Modify fields of an queue; “Queue DN” field is read-only

- Select the “radio” button for the modified agent; only one and only agent can be modified at one instance

- Click the [Update] button to proceed

Result of modify/delete-operation will be displayed in this area

## 5.3 Delete queue entry

- Check one or multiple entries to be deleted in [Delete] column

- Click the [Update] button to proceed

Result of modify/delete-operation will be displayed in this area

- Total number of records is decremented by 1

# 6. Grouped Extensions (Outbound historical reports)

Grouped Extensions refer to a named group number of a set of extensions. It is useful to produce outbound extension historical report by group of extensions. If agent ID and extension values are identical, both ACD agent and outbound extension historical reports will then matched.

Startup URL in web browser: http://<asterisk-ip>/cfgmaint

- <asterisk-ip> = IP address of Asterisk server or host name if it can be resolved in intranet environment

- Cookie must be enabled since session is required

- Username / password are required to access the web page

- Click [HELP] for online help description if needed

- Enter valid [Username] and [Password] values

- Select (x) 2. Grouped extensions configuration

- Click [Submit] button to proceed

- For fields and control button description, please return to main menu page and click [HELP]

- Since “Group name” and “Extension” are combined as a composite key, there is no modify function

## 6.1 Add group name / extension

Fields/button area to add group name/extension

- After entering values, click [Add] button to execute

Result of add-operation will be displayed in this area

If add-operation is successful, total number of records is incremented by 1

## 6.2 Delete group name/extension

- Check one or multiple entries to be deleted in [Delete] column

- Click the [Update] button to proceed

Result of modify/delete-operation will be displayed in this area

- Total number of records is decremented by 2

# 7. Inbound Routes

Inbound Routes refer to incoming call flow control from external world via TDM interface card(s), SIP User Agent or IP trunk gateway. Background information of DID (Direct Inwards Dialing) and DNIS (Dialed Number Identification Service) will be briefly introduced below.

Numbering block and DNIS

- It refers to a block of subscribed number range provided by Telco service provider.

- It is applicable to T1/E1 trunk, SIP User Agent and IP trunk gateway.

- Example of a hundred numbering block is 2345-67xx; subscribed trunk(s) receives all incoming calls in the range of 2345-6700 to 2345-6799.

- Direct Inwards Dialing (DID) extensions in this example can be any number in the range 2345-6700 to 2345-6799; length of internal extensions is part of system design subject to end-user approval.

- Number of digits sent from Telco service provider to PABX or Asterisk is configurable and should be requested to Telco service provider upon subscription order.

- The Dialed Number Identification Service (DNIS) refers to number received from Telco service provider; in FreePBX, the “DID” number is equivalent to “DNIS”.

- Any channel in a trunk group (of single or multiple T1/E1 or IP trunks) may receive any DNIS/DID value.

- Multiple hotlines / queues / applications can be configured dynamically for better resources utilization.

CO (Central Office) Analogue line or POTS (Plain Old Telephone Service)

- CO or POTS line refers to voice-grade telephone service provided for residential and small business office.

- Each line is assigned to a unique DID number.

- A group of CO analogue lines is not necessarily in continuous DID numbers.

- In some applications, Telco service provider facilitates “hunting group” service on a group of CO analogue line.

- If DNIS is required, special subscription from Telco can be made but special hardware card may be required to receive DNIS information.

- In FreePBX and Asterisk, the CO analogue can be referred by “Zaptel” channels. (NB: all channels provided by TDM cards, no matter analogue or T1/E1, are named as “Zap/” in Asterisk!)

- Configuration of multiple hotlines / queues / applications must be “hard-wired” to channel level; thus, static channels resources allocation.

Inbound route is configured in FreePBX  (Admin -> [Setup] -> [Inbound Call Control] -> [Inbound Routes]) or in /etc/asterisk/extensions.conf and /etc/asterisk/extensions_custom.conf manually. The latter will not be discussed in this document.

Startup URL in web browser: http://<asterisk-ip>/admin

- Refer to section 2 for login procedure

Note: DID extension can be configured in IAX / SIP extension configuration directly!

## 7.1 Add incoming route

Click this item to [Add Incoming Route]

- Click [Add Incoming Route] on the top of RHS

- Operation mode is changed to “Add Incoming Route”

Online help description for many fields are available when pointing cursor over the parameter field name. This document will briefly covers common fields to support normal operation. If description is left blank, the field is optional or default value.

Group: Add Incoming Route

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Description | Short meaning description of the incoming route rule |
| 2 | DID Number | Optional; received DNIS if T1/E1 or IP trunk gateway is used to match routing rule |
| 3 | Caller ID Number | Optional; caller ID number value to match routing rule |
| 4 | Zaptel Channel | Optional; routing rule is matched to specific Zap channel NB: the zapdata.conf must have “context=from-zaptel” |

Group: Fax Handling

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Fax Extension | [disabled] in pull-down list If inbound fax service is installed and configured, select dedicated extension to receive fax |
| 2 | Fax Email |  |
| 3 | Fax Detection Type | [Non] in pull-down list |
| 4 | Pause after answer | 0 |

Group: Privacy

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Privacy Manager | [No] in pull-down list |

Group: Options

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Alert Info | Leave it blank unless needed by specific requirement |
| 2 | CID name prefix | Leave it blank unless needed by specific requirement |
| 3 | Music On Hold | [default] in pull-down list |
| 4 | Signal RINGING | Leave it uncheck unless inbound calls are dropped by |

Group: Set Destination

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Queues | If selected, matching “DID Number”, “Caller ID Number” or “Zaptel Channel” incoming calls will be routed to chosen queue |
| 2 | Terminate Call | If selected, matching “DID Number”, “Caller ID Number” or “Zaptel Channel” incoming calls will be terminated with chosen treatment |
| 3 | Extensions | If selected, matching “DID Number”, “Caller ID Number” or “Zaptel Channel” incoming calls will be routed to chosen extension |
| 4 | IVR | If selected, matching “DID Number”, “Caller ID Number” or “Zaptel Channel” incoming calls will be routed to chosen IVR |
| 5 | Custom App | If selected, matching “DID Number”, “Caller ID Number” or “Zaptel Channel” incoming calls will be routed (internal Goto() function) to customer application / routine |
| … |  | Additional items will be prompted if defined |

- Click the [Submit] button at the bottom of page

- To make the change effective immediately, click [Apply Configuration Changes] at the top of page

## 7.2 Modify / delete incoming route

(2a) Current working Incoming Route

Modify parameters and values as describe in last section

(1) Click one of existing Incoming Route object on the RHS list

(2b) Current working Incoming Route

Click this area to Delete current object

- After modification or deletion, click the [Submit] button at the bottom of page

- To make the change effective immediately, click [Apply Configuration Changes] at the top of page

# 8. Outbound Routes

Outbound Routes refer to outgoing call trunk access code. In additional to provide voice access path, inbound / outbound channels / trunks allocation can be made. By default, there is an outbound route indexed 0, named “9_outside” (“0 9_outside”).

## 8.1 Trunks object

Another “Trunks” object will be referred in outbound route object; it will be briefly covered as well. A default “Trunk ZAP/g0” is predefined in FreePBX along with “0 9_outside” outbound route object.

FreePBX-> [Admin] -> [Setup] -> [Trunks]

Add ZAP Trunk: TDM card channels; to be discussed in this sub-section

Add IAX2 Trunk: tie-trunks between 2 Asterisk server; not covered in this document

Add SIP Trunk: SIP User Agent or IP trunk; not covered in this document

Add ENUM Trunk: not covered in this document

Add Custom Trunk: not covered in this document

### 8.1.1 Add Trunk

- After clicking [Add ZAP Trunk] option, the above page will be prompted

- There is object name for trunk object!

- The name is identified by [Zap Identifier (trunk name)]

Group: General Settings

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Outbound Caller ID | Leave it blank unless needed by specific requirement |
| 2 | Never Override CallerID | Leave it unchecked unless needed by specific requirement |
| 3 | Maximum channels | Leave it blank unless needed by specific requirement |
| 4 | Disable Trunk | Leave it unchecked unless needed by specific requirement |
| 5 | Monitor Trunk Failures | Leave it unchecked unless needed by specific requirement |

Group: Outgoing Dial Rules

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Dial Rules |  |
| 2 | Dial rules wizards |  |
| 3 | Outbound Dial Prefix |  |

Note: Leave all above fields blank; they will be configured in Outbound Route object.

Group: Outgoing Settings

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Zap Identifier (trunk name) Quick example of Zap channels grouping: file /etc/asterisk/zapata.conf ; context=from-zaptel switchtype = euroisdn signalling = pri_cpe group=0 channel => 1-15 group=1 channel => 17-31 | Zap channel or channels by a predefined group number or individual Zap channel identifier in /ect/asterisk/Zapata.conf Configuration of channel ‘group’ will not be covered in this paper. Channel group specification: [g¦G¦r¦R]n Where n = predefine group number, started at 0 (zero) g: select the lowest-numbered non-busy Zap channel; ascending sequential hunt group G: select the highest-numbered non-busy Zap channel; descending sequential hunt group r: use a round-robin search, starting at the next highest channel than last time; ascending rotary hunt group Note: it is, r, the suggested channel seizure algorithm R: use a round-robin search, starting at the next lowest channel than last time; descending rotary hunt group Examples: “r0” (ascending round-robin search in channel group 0) “R1” (descending round-robin search in channel group 1) Channel specification: [<span#>-]<chann#> Where <span#> is the physical T1/E1 circuit number in TDM card - (hyphen) is the delimiter if <span#> is present <chann#> is the Zap device channel number Examples: “1” (refers to Zap/1) “2-1” (refer to Zap/2-1, channel 1 in span 2) |

- Click the [Submit] button at the bottom of page

- To make the change effective immediately, click [Apply Configuration Changes] at the top of page

- new trunk “ZAP/r0” (Zap channels group 0 with ascending rotary hunt group) is added

### 8.1.2 Modify / delete Trunk

Modification or deletion of trunk object is very similar to other FreePBX objects.

- Select trunk object in the RHS list

- For modification, change option values as required

- For deletion, click the [Delete <object-name>]

- Click the [Submit] button at the bottom of page

- To make the change effective immediately, click [Apply Configuration Changes] at the top of page

## 8.2 Outbound route module

Outbound route is configured in FreePBX  (Admin -> [Setup] -> [Basic] -> [Outbound Routes]) or in /etc/asterisk/extensions.conf and /etc/asterisk/extensions_custom.conf manually. The latter will not be discussed in this document.

Startup URL in web browser: http://<asterisk-ip>/admin

- Refer to section 2 for login procedure

## 8.3 Add outbound route

Click this item to [Add Route]

- Click [Add Route] on the top of RHS

- Operation mode is changed to “Add Route”

Online help description for many fields are available when pointing cursor over the parameter field name. This document will briefly covers common fields to support normal operation. If description is left blank, the field is optional or default value.

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Route Name | Mandatory; meaningful name to describe route path / purpose |
| 2 | Route Password | Optional password for toll calls; normally, leave it blank |
| 3 | Emergency Dialing | Leave it blank for normal use |
| 4 | Intra Company Route | Leave it blank for normal use |
| 5 | Music On Hold? | [none] in pull-down list |
| 6 | Dial Patterns | A regular expression syntax string X matches any digit from 0-9 Z matches any digit form 1-9 N matches any digit from 2-9 [236-9] matches any digit or letter in the brackets (in this example, 2,3,6,7,8,9) . wildcard (period / dot), matches one or more characters ¦ (vertical bar) separate dialing prefix from number Example: 9¦[236-9]XXXXXXX matches “923456701” but passes only “23456702” for outbound dialing *Number length is limited to nine 9¦. Matches any string started with 9 and passes digits after the first “9” for outbound dialing |
| 7 | Dial patterns wizards | A set of predefined dial patterns |
| 8 | Trunk Sequence | One of more available predefined Trunk object Idle channel will be searched from top to bottom of Trunk objects. |

- Click the [Submit] button at the bottom of page

- To make the change effective immediately, click [Apply Configuration Changes] at the top of page

## 8.4 Modify / delete outbound route

(2a) Current working Outbound Route

Modify parameters and values as describe in last section

(1) Click one of existing (outbound) Route object on the RHS list

(2b) Current working Outbound Route

Click this area to Delete current object

Apply these “Move Down” and “Move Up” icons to arrange Outbound Routes matching order; e.g., prefixes are identical but patterns are different

- After modification or deletion, click the [Submit] button at the bottom of page

- To make the change effective immediately, click [Apply Configuration Changes] at the top of page

# 9. SIP Extension (optional)

SIP (Session Initiation Protocol) extension is a popular IP phone standard. Although there is no integrated soft-phone program (i.e., telephony features plus Asterisk agent mode functions) developed at this moment, it is presented here for reference. A typical use is to define SIP extension for fixed SIP hard-phone for technical support or supervisors without using a PC workstation.

SIP extension configuration in FreePBX is very similar to configuration of IAX extension. SIP extension can be configured manually in /etc/asterisk/sip_custom.conf; it is out of scope in this manual.

Startup URL in web browser: http://<asterisk-ip>/admin

- Refer to section 2 for login procedure

## 9.1 Add SIP extension

To add a new IAX extension

- Click [Extensions] module under {Basic} category

- Click [Add Extension] in the list of RHS if current mode is modification

- In the Device pull-down menu, select [Generic SIP Device]

- Click [Submit] button

Group: Add Extension

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | User Extension | Mandatory; the extension number to dial to reach this user |
| 2 | Display Name | Caller ID name (not number) for calls from this user |
| 3 | CID Num Alias | NB: leave this field blank or recorded voice file name will be incorrect for searching! |
| 4 | SIP Alias |  |

Group: Extension Options

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Direct DID | Set the Direct Inwards Direct number if this extension is configured to be reached by external party directly For normal agents, this field is left empty |
| 2 | DID Alert Info |  |
| 3 | Music on Hold | [default] in pull-down list |
| 4 | Outbound CID |  |
| 5 | Ring Time | [Default] in pull-down list |
| 6 | Call Waiting | [Disabled] in pull-down list for ACD agent’s extension |
| 7 | Emergency CID |  |

Group: Device Options

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | secret | “abc123” (the ‘default’ value in this document) |
| 2 | dtmfmode | rfc2833 |
|  | Fields/parameters in modification mode; non-default values are bolded |  |
| 3 | canreinvite | no |
| 4 | context | from-internal |
| 5 | host | dynamic |
| 6 | type | friend |
| 7 | nat | no |
| 8 | port | 5060 |
| 9 | qualify | no |
| 10 | callgroup |  |
| 11 | pickupgroup |  |
| 12 | disallow |  |
| 13 | allow |  |
| 14 | dial | SIP/nnnn where nnnn = the current extension number |
| 15 | accountcode |  |
| 16 | mailbox | nnnn@device where nnnn = the current extension number |

Group: Fax Handling

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Fax Extension | [disabled] in pull-down list |
| 2 | Fax Email |  |
| 3 | Fax Detection Type | [Non] in pull-down list |
| 4 | Pause after answer | 0 |

Group: Privacy

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Privacy Manager | [No] in pull-down list |

Group: Recording Options

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Recording Incoming | Select [On Demand], [Always] or [Never] as required |
| 2 | Recording Outgoing | Select [On Demand], [Always] or [Never] as required |

Group: Voicemail & Directory

| No. | Parameter name | Description |
| --- | --- | --- |
| 1 | Status | [Enabled] in pull-down list to create mailbox for the extension; otherwise, [Disabled] |
|  |  | If [Enabled], set below parameters |
| 2 | Voicemail Password | Password to access the voice mail system; must be all numbers |
| 3 | Email Address | Optional email address to get voice mail notification |
| 4 | Pager Email Address | Optional pager email address to get voice mail notification |
| 5 | Email Attachment | “yes/no”; if email address is given, recorded voice mail file can be attached in the voice mail notification |
| 6 | Play CID |  |
| 7 | Play Envelope |  |
| 8 | Delete Vmail | “yes/no”; if email address is given, it controls if the voice mail should be deleted after sending notification mail. NB: the option does not check if “Email Attachment” is set! |
| 9 | VM options | Additional voice mail options separate with pipe (¦) Options specified here may override global values maxmessage=180 ; max seconds of vmail message minmessage=3 ; min seconds of vmail message maxsilence=8 ; Wait for 8 silent seconds before ending the voicemail recording silencethreshold=128 ; silence threshold value skipms=3000 ; How many milliseconds to skip forward/back when rew/ff in message playback review=no ; Allow sender to review/rerecord their message before saving it operator=no ; Allow caller to press 0 nextaftercmd=yes ; Automatically play next message when current message is deleted or saved |
| 10 | VM Context | Leave it as “default”, the default value |
| 11 | VM X LocatorTM | [Disabled] in pull-down menu |

Click the [Submit] button at the bottom of page

To make the change effective immediately,

Click [Apply Configuration Changes] at the top of page; the “Apply” button is usually clicked after finishing a batch of changes.

In the window, “Reloading will apply all configuration changes made in FreePBX to your PBX engine and make them active”, click [Continue with reload] to proceed.

## 9.2 Modify / Delete SIP extension

(2a) Current working extension

Modify parameters and values as describe in last section

(1) Click one of existing extension on the RHS list (not shown in this diagram!)

(2b) Current working extension

Click this area to Delete the current extension

- After modification or deletion, click the [Submit] button at the bottom of page

- To make the change effective immediately, click [Apply Configuration Changes] at the top of page

# 10. Features codes configuration

A list of built-in feature codes is preset by FreePBX. Some of them are commonly used in typical call center environment. All will be briefly discussed in this section.

The page is accessed in [Admin] -> [Setup] -> (Basic) -> [Feature Codes]

Continued…

Notes:

- Uncheck the checkbox “Use Default?” to remove preset feature code

- Select [Disabled] at the RHS of a feature code to disable them

- If default feature code is changed, please make sure that new code does not conflict with any existing feature and outbound prefix!

- Day/Night Mode requires separate configuration.

- ChanSpy (555) will not work if you are Record()-ing the spied channel; unless you add transmit_silence_during_record=yes to the options section in /etc/asterisk/asterisk.conf.

- The current Asterisk version is 1.2.28.1; up to and including Asterisk 1.4.17 ChanSpy can cause a crash/segfault if it is used together with Monitor or MixMonitor at the same time. 1.4.18 is supposed to attack this issue by using "audiohooks" that replaces the current ChanSpy approach.

  - Thus, the phone extension spying other extensions should have monitoring feature TURNED OFF.

- Important note: the default 555 feature code generated by FreePBX does not allow selection of particular extension (e.g., nnnn#); the “go to next” command “*” works as expected. Refer to next section for workaround solution.

- After modification or deletion, click the [Submit] button at the bottom of page

- To make the change effective immediately, click [Apply Configuration Changes] at the top of page

## 10.1 Silence monitor customization

The default 555 feature code generated by FreePBX does not allow selection of particular extension (e.g., nnnn#); the “go to next” command “*” works as expected. This section is added to provide workaround solution.

    - Edit Asterisk configuration file /etc/asterisk/extensions_custom.conf

vi /etc/asterisk/extensions_custom.conf

    - Add below dial-plan statements within content [from-internal-customer]

[from-internal-custom]

include => from-internal-trixbox

exten => 557,1,Noop(Spy IAX device)

exten => 557,n,Answer

exten => 557,n,Wait(1)

exten => 557,n,ChanSpy(IAX2)

exten => 557,n,Hangup

    - Reload change in Asterisk CLI

asterisk *CLI> extensions reload

    - Verify change in Asterisk CLI

asterisk *CLI> show dialplan  from-internal-customer

'557' =>          1. Noop(Spy IAX device)                       [pbx_config]

2. Answer()                                   [pbx_config]

3. Wait(1)                                    [pbx_config]

4. ChanSpy(IAX2)                              [pbx_config]

5. Hangup()                                   [pbx_config]

The new customized feature code 557 is created for silence monitory with jump-to IAX2 extension number capacity. If SIP extension is implemented, separate feature code is required. Example is illustrated below.

exten => 559,1,Noop(Spy SIP device)

exten => 559,n,Answer

exten => 559,n,Wait(1)

exten => 559,n,ChanSpy(SIP)

exten => 559,n,Hangup

# Appendix A – Configuration Tables Maintenance Data Flow Diagram

- Agent Profile Data Flow Diagram

Display agent profile

Add agent

Modify agent(s)

Delete agent(s)

Upload to Asterisk button clicked

cfgagent MySQL database table

agents_external.conf Asterisk external agent file

Add button clicked

Update button clicked

Confirm button clicked

Read cfgagent table

Write configuration file

Reload  Asterisk chan_agent.so

- Queue (Service Level) Attributes For Historical Reports Data Flow Diagram

Display queue profile

Add queue

Modify queue(s)

Delete queue(s)

cfgqueue MySQL database table

Add button clicked

Update button clicked

- Grouped Extensions (Outbound Historical Reports) Data Flow Diagram

Display grouped extensions

Add grouped extensions

Delete grouped extensions

groupedExtTbl MySQL database table

Add button clicked

Delete button clicked