IAX Soft-phone

User Guide

29 Oct 2008                                                                                                                    Version 1.08

Sunny Line Limited © 2007-2008

Trademarks

Digium and Asterisk are registered trademarks of Digium, Inc.

http://www.digium.com

FreePBX is a registered trademark of Atengo LLC.

http://www.freepbx.org

Microsoft, Windows, Window 2000 / 2000 Professional / XP Home / XP Professional / 2003 server MS SQL 2005 Server, MS Visual Studio 2005 and their components are registered trademarks of Microsoft Corporation.

http://www.microsoft.com

All other trademarks are property of their respective owners.

Table of Content

1. Introduction	1

1.1 Viewing Runtime Parameters	2

2. Starting / terminating iaxSphone.exe Soft-phone	4

2.1 IAX User/Extension Registration	6

2.2 Terminating Soft-phone	9

3. Telephony Soft-phone Functions	11

3.1 Make external / internal outgoing call	11

3.2 Receive external / internal incoming call	14

3.3 Unattended Call Transfer	18

3.3.1 Attended Call Transfer via Asterisk feature code	19

4. Agent Mode functions	20

4.1 Agent Login/Logout	21

4.2 Agent modes	22

4.3 Timed ACW and other configurable Agent status	24

4.4 Message Waiting Indicator (MWI)	25

5. Opt-out Number Validation	26

5.1 Opt-out database/table connection exception	28

# 1. Introduction

This document describes IAX2 soft-phone features and operations. Program startup, operations and termination will be presented with examples. Workstation running the iaxSphone.exe should be on Microsoft Window platform; Window 2000/Professional, Window XP Home SP1/2 and Window XP Professional SP 2.

Configuration startup file of iaxSphone.exe should have been prepared by system support staff or software supplier. Configurable parameters are described in document “IAX Soft-phone configuration guide”.

Topics covered in this document include:

- Starting/Terminating iaxSphone.exe

  - Indicator of Opt-out / Do-not-call features

  - IAX user/extension registration

- Soft-phone functions

  - Make external / internal outgoing call

  - Receive external / internal incoming call

  - Hold / unhold call

  - Release call

  - Unattended call transfer

    - Attended call transfer via Asterisk access code

  - Inbound call auto answer / force-in

- Agent mode functions

  - Agent login / logout into/from queue

  - Ready

  - Not Ready

  - After Call Work – Inbound (report messages; Asterisk agent mode as Not Ready)

  - After Call Work – Outbound (report messages; Asterisk agent mode as Not Ready)

  - Timed ACW (software implementation)

  - Not Ready – Outbound call in progress

  - Not Ready – Inbound call in progress

  - Not Ready with reason code (AUX6 to AUX11)

- Mail Waiting Indicator

  - MWI polling; must be in logged-in state

- Opt-out numbers validation

  - Optional runtime outbound call opt-out table lookup

Target audiences are call center supervisors and agents.

## 1.1 Viewing Runtime Parameters

Runtime parameters and values can be verified in [About iaxSphne…] item, by clicking at the upper left corner icon. Information is helpful for settings verification.

# 2. Starting / terminating iaxSphone.exe Soft-phone

Your local system support or software supplier should have prepared working configuration file and a shortcut file on desktop area or dedicated folder to start the iaxSphone.exe. The program is started by double-clicking the shortcut file.

A

B

C

D

E

F

G

H

I

C

[A]: Opt-out enablement indicator; if disabled, titled “IAX2 Soft Phone <version>” and no (xxxx.ini)

[B]: Telephony function buttons; available after successful IAX user registration

[C]: Agent mode buttons; available after successful IAX user registration and agent login

[D]: Inbound call information; Caller Line Identification and Inbound queue/hotline

[E]: IAX user registration, agent login/logout and program termination buttons

[F]: IAX user/extension registration fields; they cannot be alternated after STARTED

[G]: Required fields for agent login

[H]: Speaker and Microphone volume controls; percentage of window current settings

[I]:   Keypad to generate DTMF in IVRS or voice interaction requiring it

Telephony control buttons are enabled / disabled according to registration and line states. Below diagram summaries their relationships.

Hold/Unhold

Release

Start

Dial

Answer

Login

Ready

AUX

ACWIB

ACWOB

IAX registered

(Line idle)

Agent logged in

Logout

Line in used

Single Txfer

Outbound NR

Inbound NR

Training

Coaching

Briefing

Admin work

Pre call work

15-min break

Pull-down menu

Top two items are fixed; other six captions are configurable.

Default captions are AUX6, AUX7,…, AUX11

Control buttons calling orders and flows

## 2.1 IAX User/Extension Registration

F

Three mandatory parameters are required for IAX Extension / User registration before any telephony basic functions.

[Host]: IP address or host name (if resolvable by workstation / network) of Asterisk server

[Ext/User]: Extension / user name of registration

[Password]: Password for user registration

- Click [Start] button to register

- If connected,

  - the [Start] button will be changed to [ Online ]

  - [Dial] and [Login] buttons will be activated

If extension verification option is enabled, extra verification steps will be executed. Possible messages are listed if exceptions are detected.

- The extension [3090] is not a valid extension; program cannot be started

- Check the extension number, change it and try again

- Consult local IT support if necessary

- The extension [3001] is found being used in other workstation; program cannot be started

- Check the extension number, change it and try again

- Consult local IT support if necessary

- The extension [3001] is found being used in the local workstation.

- Case occurs when iaxSphone program with the same extension in the same workstation is stopped and restarted within 1 minute.

- Click [Yes] if the single iaxSphone program is run on same workstation with same extension

- Click [No] to abort registration otherwise

- Consult local IT support if necessary

After successful registration, inbound call (to extension/user 3001 in this example) can be received.

If “loggedInToDial” option is false/no, outbound call can be initiated without agent login.

Otherwise, following message will be prompted.

Note: if the                      button changed to                     , it means the soft phone can’t connect to Asterisk server. The cause of problem may be network connectivity problem, incorrect host name, ext/user, password and etc. User should consult system support / administrator when encounter this problem.

## 2.2 Terminating Soft-phone

The iaxSphone.exe program can be terminated by clicking [Exit] button or “x” icon at the upper right corner of window. There is no deregistration function; user needs to terminate current extension/user registration and starts soft phone again to register new extension/user.

- If there in no active conversation, above confirmation box will be prompted.

- Click [Yes] to confirm program termination

  - Agent will be logged out by program if already logged in

  - Extension/user will be deregistered if already registered

- Click [No] to cancel termination

If there is active conversation on the soft phone, above message box will be prompted; user may complete current conversation before terminating the soft phone.

# 3. Telephony Soft-phone Functions

Telephony functions such as making outbound call, hold/unhold, release, single-step transfer and receiving incoming call will be illustrated in the section.

## 3.1 Make external / internal outgoing call

- Select the textbox on the RHS (Right Hand Side) of [Dial] button

- Enter outgoing call number

  - for external call type, corresponding access code  may be required depending on specific call center configuration; consult system administrator for access code prefix making external outbound call type

- Click [Dial] button

- If circuit is available and connected (staring from ringing), [Release], [Single Txfer] and [Hold] buttons will be enabled

- Click [Hold] button to hold the active interaction; other party (the called party in this case) will be hold and hears music-on-hold if configured.

- The voice interaction is current put on hold; the [Hold] button caption is changed to [Unhold]

- Click [Unhold] button to retrieve the held interaction

- The held interaction is retrieved; conversation is resumed

- Click [Release] button to terminate current voice interaction

- Previous voice call is released and soft phone extension is back to idle

- If other party releases the call and release-signal is present, soft phone extension state will be set to idle automatically

## 3.2 Receive external / internal incoming call

When soft phone extension / user state is idle and incoming call arrives, the following events follow

- iaxSphone.exe program window will be put in front of all active windows

- Calling extension (internal call) or caller line identifier (external call) will be shown in “Remote Number” textbox

- Textbox “Remote content” displays last Asterisk Content or object to this extension

- For manual answer configuration as in this example

  - Ringing tone will be heard in the speaker device

  - The [Answer] button will be flashed as [ Answer ] and [ Answer ].

  - Incoming will not be answered until [Answer] button is pressed

- For auto-answer configuration,

  - An incoming alert around half second will be generated

  - Call will be answered after the incoming call alert

- Incoming call arrives at extension with manual answer

- Ringing tone will be heard in speaker device

- Color of the [Answer] button is changed

- If incoming call is not answered,

- Ringing tone will be continued in speaker device

- Color of the [Answer] button will be changed to red; green-red alternation along ringing tone

- Click the [Answer] button to take the incoming call

- Incoming call is connected; [Hold], [Single Txfer] and [Release] buttons are enabled when in conversation

- For normal inbound application or outbound without using DTMF keypad, lower half window portion can be hidden by the resize button [ << ] (to hide lower portion) or [ >> ] (to show lower portion)

- Click [ Release ] button to release current interaction

- Extension / user state is back to idle

In this example, user is logged into queue 8001 and agent mode is ready.

- An incoming call arrives; CLI is “3901” (internal extension number) and Remote content is “Q8001sip3019” (from user sip3901 via Q8001)

- Click [Answer] to take the call

- After finishing the  conversation, wait caller to hang up or click [Release] to terminate the call

- Extension / user state is back to idle; however, agent mode is “ACWIB”!

## 3.3 Unattended Call Transfer

Unattended call transfer (or single-step / blink transfer) function is to route active interaction to a destination with supervisor or “hand-shaking” to the transferred party. If the transferred party is an extension, being in used and no call waiting feature, the transferred call will be dropped.

Typical use of unattended call transfer is to transfer active interaction to another queue device or IVR group number that has call queuing capacity.

- Incoming call arrives; click [Answer] button to take it

- Enter transferred destination in the textbox RHS of [Single Txfer] button

- Click [Single Txfer] button to perform unattended transfer

- Active interaction is transferred to destination “8002”

### 3.3.1 Attended Call Transfer via Asterisk feature code

Current version of iaxSphone.exe does not implement attended call transfer at IAX protocol 2 level. If attended call transfer is required, Asterisk switch-level attended call transfer should be used instead. By default, two feature codes are enabled for attended call transfer. If there access codes do not work (e.g., T and/or t options must be configured in command Dial), please consult system support/administrator or software supplier.

*2 Attended call transfer initiation

*0 Disconnect for attended transfer (default ** code is conflicted to FreePBX pickup code)

Extension is in used (active conversation)

Other party will be put on hold

Agent hears announcement “Transfer”

*2

nnnn#

Transferred number invalid / busy

Transferred party rings

Transferred party answers call

Transferred party hangs up

*0 (states 1, 2, 3)

State 4

Call transfer completes

Extension is back to idle state

[Release]

- While on conversation with another party, user dials the attended transfer key sequence *2.

- Asterisk says "Transfer" then gives a dial tone

- While putting the other party on hold, user dials the transferee number (nnnn, followed by end of input character #).

- If transferred number is invalid/busy, in ringing or conversation with transferee party, user may enter disconnect code *0 to return to the original held party.

- If the transferee answers the call and hangs up, user will be connected back to original conversation.

- If user clicks [Release] button, the call transfer completes and extension/user is back to idle state.

Note: The caller ID presented to the transferred party is the user (the one initiates the call transfer) caller ID. The original caller ID will not be propagated to the transferred party.

# 4. Agent Mode functions

Agent modes are primary for inbound application and for working with Automated Call Distribution (ACD) device such queue. Agent mode functions include login, logout, ready, ACW (after call work, call related Not-Ready) and AUX (auxiliary, non-call related Not-Ready).

| No. | State | Description |
| --- | --- | --- |
| 1 | Logged out | Agent is not registered in the queue to take inbound call |
| 2 | Logged in | Agent is registered in the queue but not necessarily able to take inbound call; it depends on Agent status |
| 3 | Ready | Agent is ready to take inbound call |
| 4 | AUX | Agent is Not Ready to take inbound call; busy for non-call related work or short-break |
| 5 | ACWIB | Agent is Not Ready to take inbound call; busy for last inbound call wrap-up work |
| 6 | ACWOB | Agent is Not Ready to take inbound call; busy for last outbound call wrap-up work |
|  |  | Pull-down list |
| 7 | Outbound NR | Not Ready due to outbound call in progress; controlled by ‘outboundDialNotReady’ option. Agent cannot set it manually |
| 8 | Inbound NR | Not Ready due to inbound call in progress; controlled by ‘inboundAnswerNotReady’ option. Agent cannot set it manually |
| 9 | AUX6 | Not Ready with configurable caption; set to ‘Training’ in this example |
| 10 | AUX7 | Not Ready with configurable caption; set to ‘Coaching’ in this example |
| 11 | AUX8 | Not Ready with configurable caption; set to ‘Briefing’ in this example |
| 12 | AUX9 | Not Ready with configurable caption; set to ‘Briefing’ in this example |
| 13 | AUX10 | Not Ready with configurable caption; set to ‘Briefing’ in this example |
| 14 | AUX11 | Not Ready with configurable caption; set to ‘Briefing’ in this example |

Notes:

- AUX / ACWIB / ACWOB / Outbound NR / Inbound NR / AUX6…AUX11 agent status are for historical reports only; to Asterisk, these status are the same as Not-Ready (paused in Asterisk queue term)

- ACWOB / Outbound NR / Inbound NR are not standard agent status in typical ACD switch; it is customized to generate After-Call-Work and other statistics in historical reports.

## 4.1 Agent Login/Logout

The [Login] button is enabled if and only after successful extension/user registration; The [Logout] button is enabled after successful agent logged in session.

Click [Login] to log-in into queue device

The logout button

Agent status buttons after logged in; dimmed or disabled button is the current status

## 4.2 Agent modes

Current status is “Ready”

Click [AUX]

Current status is “AUX”

Click [ACWIB]

Current status is “ACWIB”

Click [ACWOB]

Current status is “ACWOB”

Starting at version 1.0.1.0, additional “Not Ready” with configurable reason codes are available. Agent may select them from the pull-down list shown above.

The first two item “Outbound NR” and “Inbound NR” are read-only entries; they are controlled by inbound/outbound action options.

If agent selects “Outbound NR” or “Inbound NR”, a warning message box will be prompted

These 6 caption-configurable Not-Ready modes behave identical to [AUX] / [ACWIB] / [ACWOB]; they are selectable from pull-down list rather than buttons.

## 4.3 Timed ACW and other configurable Agent status

Timed ACW (After-Call-Work) is a function to set agent status to ACW for a configured interval and resume agent status to READY automatically after the configured interval. If agent manually changes agent status within the interval (e.g., Ready or AUX), the timed function will be cancelled. It is normally applicable to inbound call type.

There is a set of runtime parameters affecting agent status; please refer to section 1.1. Brief information will be presented below; consult system administrator or software supplier for details.

Agent mode operations parameters

| No | Parameter | Description |
| --- | --- | --- |
| 1 | loginAction | Default is 0 (Not Ready) - agent mode initialization after login - Valid values: 0 = Not Ready, 1=Ready, other=do nothing |
| 2 | noChangeAgentWhenBusy | Default is 1/yes (NOT allow changing agent mode when in-use) - If set to 0, agent mode can be changed when line is busy or in use; otherwise, warning message box will be prompted |
| 3 | inboundAnswerNotReady | Default is 1/yes (Inbound Not-Ready) - It controls if Not-Ready agent mode will be sent after answering inbound calls - Valid values: non-zero=Not Ready, zero=no action |
| 4 | outboundDialNotReady | Default is 1/yes (Outbound Not Ready) - It controls if Not-Ready agent mode will be sent before making outbound call - Valid values: non-zero=Not Ready, zero=no action |
| 5 | inboundReleaseAction | Default is 0/No action - It controls agent mode after releasing inbound call type - Valid values: 0=No action, 1=Ready, 2=Not Ready, 3=ACW, 4=Timed ACW, others=no action |
| 6 | inboundTimedACW | Default is 0 (disabled; i.e., inboundReleaseAction=0) - It specifies timed ACW (After-Call-Work) duration in second for inbound call type - It is effective if inboundReleaseAction=4 - Valid range is 1...1000 (second) |
| 7 | outboundReleaseAction | Default is 0/No action - It controls agent mode after releasing outbound call type - Valid values: 0=No action, 1=Ready, 2=Not Ready, 3=ACW, 4=Timed ACW, others=no action |
| 8 | outboundTimedACW | Default is 0 (disabled; i.e., outboundReleaseAction=0) - It specifies timed ACW (After-Call-Work) duration in second for outbound call type - It is effective if outboundReleaseAction=4 - Valid range is 1...1000 (second) |

## 4.4 Message Waiting Indicator (MWI)

The iaxSphone.exe can be configured to associate to a voice mailbox number. As at this stage, the MWI feature is valid only agent logged-in state.

- If voice mailbox is enabled and there is new message(s), the “new message” button will be shown.

- Polling method is used to acquire unread new message(s).

- If new message is read / handled, the “new message” button will disappear at next polling interval.

- The “new message” (MWI) button appears if there is new voice message in the specified mailbox

- Click the button to verify

- In this example, the voice mailbox number [3901] has 4 new voice mails

Notes:

- If new voice message(s) is retrieved, the MWI button stays until next polling interval; default is 300 seconds (5 minutes).

- After manual clicking the MWI button and if there is new message count being zero, the MWI button will disappear.

# 5. Opt-out Number Validation

Opt-out or Do-Not-Call (DNC) number validation is applied to outbound call. General practice is to have filtering process DURING loading customer calling records into customer database table; this process is out of scope of this document. This online opt-out number validation is optional to catch last-minute update opt-out numbers.

If this option is enabled, proper system and program configuration should have done by system support/administrator or software supplier. Soft phone users should consult system support / administrator for site-specific setup.

If opt-out number validation is enabled, the “campaign” level configuration file name will be shown at the upper left window title bar.

- In this example, the opt-out number validation profile is set to “optout.ini”.

- Click [Start] to perform extension/user registration

- Click [Login] afterwards to log in agent

- Click the [ << ] resize button to hide lower portion

- Enter outgoing calling number in the textbox RHS to [Dial] button

- Click [Dial] button

- The outgoing number dialed number “92303311” is found in the opt-out table

- Click [OK] to return

## 5.1 Opt-out database/table connection exception

If database server is out of service due to network connection or database server itself, opt-out number validation will be failed. Window box will be prompted for user action; two types of content are possible, depending on program status; “dbOpenConnection failed!” or “Database/table retrieval failed!”.

- If [Cancel] button is clicked, the current number is NOT dialed and Opt-out status stays active

- The opt-out profile name is kept active as shown at the upper left window title bar.

- It is the default button focus when window prompt appears.

When opt-out database server is out of service, agents should consult system support / administrator. Whether continuing to make outbound calls without opt-out number validation is campaign-level specific requirement.

- If [Yes] button is clicked, the current number is dialed and Opt-out status will be set to disabled afterwards

- The opt-out profile name disappears at the upper left window title bar.

- If [No] button is clicked, the current number is dialed and Opt-out status stays active

- The opt-out profile name is kept active as shown at the upper left window title bar.